ESE 499 Undergraduate Research –Summer 2026

Real-Time FM Audio Wireless Transceiver System Using PlutoSDR

Name: Sohee Kim

Student ID: 117643907

Date: June11, 2026

Email: sohee.kim.3@stonybrook.edu

Advisor: Prof. Harbans Dhadwal

- 1. System Architecture & Setup

This project implements a fully functional, over-the-air Frequency Modulation (FM) wireless audio broadcasting and reception loop using two Analog Devices ADALM-PLUTO (PlutoSDR) platforms. The transmitter runs on a Linux (Ubuntu) environment, while the receiver operates dynamically under Windows 11 via Python baseband signal processing pipelines.

| - TX Station (Linux/Nano) [music.wav] -> FM Baseband Fs: 1 MHz - Carrier Frequency: 915.0 Hardware Attenuation: -20 dB | - Over-the-Air | - RX Receiver (Windows/PS) Quadrature Phase Discriminator - Decimation: 1 MHz -> 44.1 kHz - Carrier Frequency: 915.0 MHz - Hardware RX Gain: 20 dB |
| --- | --- | --- |
|  | - SMA/RF Link |  |

- Hardware Interconnection Specifications

- RF Link Type: Coaxial SMA Cable Direct Connection (Line-of-Sight Emulation)

- Port Mapping: Linux Transmitter TX1 Port $\rightarrow$ Windows Receiver RX1 Port

- Power & Gain Optimization: Adjusted to prevent analog-to-digital converter (ADC)

- 2. Transmitter (TX) Source Code

- Filename: pluto_music_tx.py

- Environment: Linux (Ubuntu) Terminal / nano editor

import numpy as np import adi import sys from scipy.io import wavfile # 1. Load the converted WAV file via SciPy wav_filename = "music.wav" try: print(f"[AUDIO] Reading '{wav_filename}' into DSP pipeline...") audio_fs, audio_data = wavfile.read(wav_filename) # Normalize integer PCM data to float ranges between -1.0 and 1.0 if audio_data.dtype == np.int16: audio_data = audio_data / 32768.0 elif audio_data.dtype == np.int32: audio_data = audio_data / 2147483648.0 print(f"[AUDIO] Success! Loaded {len(audio_data)/audio_fs:.1f} seconds of music.") except FileNotFoundError: print(f"[ERROR] Could not find '{wav_filename}'.") print("[ERROR] Please run the ffmpeg command in your terminal first!") sys.exit(1) # 2. Perform FM Modulation (Audio to Complex IQ Vector Mapping) kf = 12000 # Frequency deviation constant matching the Windows RX specs exactly phase = 2 * np.pi * kf * np.cumsum(audio_data) / audio_fs tx_iq_signal = np.exp(1j * phase) * 0.5 # Safe hardware amplitude scaling to prevent distortion tx_iq_signal = tx_iq_signal.astype(np.complex64) # 3. Configure PlutoSDR Hardware Specifications for Continuous Broadcasting try: sdr = adi.Pluto() except Exception as e: print("[ERROR] Cannot find PlutoSDR hardware on Linux USB port.") sys.exit(1) sdr.sample_rate = int(1e6) # TX Baseband Sampling Rate: 1 MHz sdr.tx_lo = int(915e6) # Target RF Carrier Frequency: 915.0 MHz (ISM Band) sdr.tx_hardwaregain_chan0 = -20 # Attenuated to -20 dB to prevent RX saturation over direct cable link print(f"\n[TX] 📡 Now broadcasting '{wav_filename}' over the air!") print(f"[TX] 📡 Target Carrier Frequency: {sdr.tx_lo/1e6:.1f} MHz") print("[TX] 📡 The music will loop continuously. Press Ctrl+C to shut down your station.") # 4. Push music IQ samples into PlutoSDR internal hardware buffer for cyclic playback try: sdr.tx_cyclic_buffer = True # Enable hardware memory buffering loops for seamless streaming sdr.tx(tx_iq_signal) # Initiate broadcasting loop onto the RF front-end while True: # Hold the execution thread open to sustain continuous cyclic streaming pass except KeyboardInterrupt: print("\n[TX] Shutting down the PlutoSDR radio station transmission...") sdr.tx_destroy_buffer()

- 3. Receiver (RX) Source Code

- Filename: pluto_radio_rx.py

- Environment: Windows 11 PowerShell

import os import sys import ctypes # Force Windows system to extend its core DLL search path for libiio binding if sys.platform == "win32": try: # Bind standard Windows system DLL directory to prevent NoneType environment bugs ctypes.windll.kernel32.SetDllDirectoryW(None) except Exception: pass import numpy as np import adi import sounddevice as sd # 1. Initialize PlutoSDR Hardware Specifications for RX Receiver try: sdr = adi.Pluto() except Exception as e: print("[ERROR] Cannot find PlutoSDR hardware on Windows USB port.") print("[ERROR] Hardware binding failed. Please reconnect the USB cable.") sys.exit(1) # Configure PlutoSDR RF parameters to match the transmitter (TX) specifications sdr.sample_rate = int(1e6) # RX Baseband Sampling Rate: 1 MHz sdr.rx_lo = int(915e6) # Target RF Carrier Frequency: 915.0 MHz (ISM Band) sdr.gain_control_mode_chan0 = 'manual' sdr.rx_hardwaregain_chan0 = 20 # Attenuated to 20 dB to safely digitize direct SMA input # Define digital signal processing (DSP) and acoustic variables audio_fs = 44100 # Standard audio playback sampling rate kf = 12000 # Frequency deviation constant matching TX specs decimation_rate = int(sdr.sample_rate / audio_fs) print(f"\n[RX] 📻 Radio Receiver Initialized at {sdr.rx_lo/1e6:.1f} MHz") print("[RX] 📻 Real-time audio stream binding... Press Ctrl+C to mute.") # 2. Open Continuous Audio Stream Buffer via Sounddevice (Non-blocking Callback approach) stream = sd.OutputStream(samplerate=audio_fs, channels=1, dtype='float32') stream.start() # 3. Real-time Quadrature Demodulation and Streaming Loop try: while True: # Buffer capture: Pull high-speed Complex IQ samples from PlutoSDR hardware memory rx_samples = sdr.rx() # Baseband FM Demodulation: Differentiate the unwrapped phase variations over time # This acts as a mathematical discriminator retrieving audio from the complex phase domain demodulated_raw = np.diff(np.unwrap(np.angle(rx_samples))) # Downsample the 1 MHz raw data rate down to standard 44.1 kHz audio speed audio_out = demodulated_raw[::decimation_rate] # Amplify the demodulated audio signal to pierce through the channel white noise audio_out = audio_out * 5.0 audio_out = np.clip(audio_out, -1.0, 1.0) # Hard limiter to prevent digital clipping # Playback: Push the processed floating-point audio slice into Windows sound card queue stream.write(audio_out.astype(np.float32)) except KeyboardInterrupt: print("\n[RX] Muting the radio receiver and releasing hardware hooks...") stream.stop() stream.close()

- 4. Experimental Results & Troubleshoots

- During development, multiple technical bottlenecks regarding the Windows software environment and the RF physical layer were systematically debugged and resolved:

- 1) Windows Path Environment Resolution (NoneType Error)

- Problem: Windows Python environment initially failed to locate the core hardware abstraction engine, throwing a TypeError: argument of type 'NoneType' is not iterable upon executing import adi.

- Solution: Installed the official standard binary package (libiio-setup-v0.24-x64.exe) to register the core libraries into the Windows system space. Integrated the ctypes.windll.kernel32.SetDllDirectoryW(None) interface in python to force dynamic runtime binding.

- 2) RF Over-the-Air Channel Optimization & Attenuation

- Problem: Operating under wireless bar antennas resulted in low Signal-to-Noise Ratio (SNR), causing severe white noise distortion due to multi-path fading within the indoor workspace.

- Solution: Replaced the wireless channel with a physical coaxial SMA cable link bridging the Linux TX1 and Windows RX1 ports. To mitigate hardware front-end saturation (clipping), the TX hardware attenuation was throttled to -20 dB and the RX gain adjusted to 20 dB, resulting in a crisp, crystal-clear real-time audio playback through the PC sound card.